PRELIMINARY TASK – ARDUINO AND GIT INSTALLATION [5 MARKS]

(a) Create a GitHub account.

(b,c)Create a repository where you will work and include the files for this project.

(b)

(c)

clear; clc; close all;

arduinoPort = 'COM3';

arduinoBoard = 'Uno';

try

a = arduino;            % Create Arduino communication object

disp('The Arduino connection was successful!');

catch ME

error('Arduino connection failed.');

end

ledPin = 'D12';             % LED digital pin (flashing test)

blinkCount = 10;

disp(['Start flashing LED，A total of ', num2str(blinkCount), ' times.']);

for i = 1:blinkCount

writeDigitalPin(a, ledPin, 1);  % Turn on LED (high level)

pause(0.5);

writeDigitalPin(a, ledPin, 0);  % Turn off LED (low level)

pause(0.5);

end

clear a;

disp('The flashing is complete. The Arduino connection has been closed!');

TASK 1 – READ TEMPERATURE DATA, PLOT, AND WRITE TO A LOG FILE [ 20 MARKS]

clear; clc; close all;

duration = 600;

sample_interval = 1;

sensor_pin = 'A1';           % Temperature sensor analog pin (MCP9700A)

arduino_port = 'COM3';

arduino_board = 'Uno';

V_0C = 0.4;                  % MCP9700A voltage at 0°C (V)

T_C = 0.01953;               % MCP9700A temp coefficient (V/°C)

try

a = arduino(arduino_port, arduino_board);

disp(' The Arduino connection was successful!');

catch ME

error('Arduino connection failed.');

end

num_samples = duration / sample_interval;

time_vec = 0:sample_interval:duration-sample_interval;

temp_vec = zeros(1, num_samples);

disp([' Start collecting. ', num2str(duration/60), 'min temperature data']);

for i = 1:num_samples

V_out = readVoltage(a, sensor_pin);    % Read sensor voltage

temp_vec(i) = (V_out - V_0C) / T_C;    % Convert voltage to temperature

pause(sample_interval);

end

disp(' Data collection completed! ');

max_temp = max(temp_vec);

min_temp = min(temp_vec);

avg_temp = mean(temp_vec);

figure('Color','w');

plot(time_vec/60, temp_vec, 'b-o', 'LineWidth', 1.5, 'MarkerSize', 4);

xlabel('Time (minutes)', 'FontSize', 12);

ylabel('Temperature (°C)', 'FontSize', 12);

title(['Temperature vs. Time (', num2str(duration/60), ' minutes)'], 'FontSize', 14);

grid on;

saveas(gcf, 'temperature_plot.png');  % Save temperature curve to file

disp(' The temperature curve has been saved as temperature_plot.png');

current_date = datestr(now, 'dd/mm/yyyy');

location = 'Nottingham';

fprintf('Data logging initiated - %s\n', current_date);

fprintf('Location - %s\n\n', location);

max_minute = ceil(duration/60);

for minute = 0:max_minute

idx = minute * 60 + 1;

if idx > num_samples

temp_val = temp_vec(end);

else

temp_val = temp_vec(idx);

end

fprintf('Minute\t\t%d\n', minute);

fprintf('Temperature\t%.2f C\n\n', temp_val);

end

fprintf('Max temp\t%.2f C\n', max_temp);

fprintf('Min temp\t%.2f C\n', min_temp);

fprintf('Average temp\t%.2f C\n\n', avg_temp);

fprintf('Data logging terminated\n');

fid = fopen('capsule_temperature.txt', 'w');  % Open log file (write mode)

if fid == -1

error(' The log file cannot be created!');

end

fprintf(fid, 'Data logging initiated - %s\n', current_date);

fprintf(fid, 'Location - %s\n\n', location);

for minute = 0:max_minute

idx = minute * 60 + 1;

if idx > num_samples

temp_val = temp_vec(end);

else

temp_val = temp_vec(idx);

end

fprintf(fid, 'Minute\t\t%d\n', minute);

fprintf(fid, 'Temperature\t%.2f C\n\n', temp_val);

end

fprintf(fid, 'Max temp\t%.2f C\n', max_temp);

fprintf(fid, 'Min temp\t%.2f C\n', min_temp);

fprintf(fid, 'Average temp\t%.2f C\n\n', avg_temp);

fprintf(fid, 'Data logging terminated\n');

fclose(fid);  % Close log file (save data)

disp(' The log has been written capsule_temperature.txt');

clear a;

disp(' The Arduino connection has been closed!');

Answer:

Data logging initiated - 13/03/2026
Location - Nottingham

Minute		0
Temperature	16.56 C

Minute		1
Temperature	16.31 C

Minute		2
Temperature	16.56 C

Minute		3
Temperature	16.31 C

Minute		4
Temperature	16.56 C

Minute		5
Temperature	16.56 C

Minute		6
Temperature	16.81 C

Minute		7
Temperature	16.06 C

Minute		8
Temperature	16.56 C

Minute		9
Temperature	16.31 C

Minute		10
Temperature	16.31 C

Max temp	16.81 C
Min temp	16.06 C
Average temp	16.41 C

Data logging terminated

TASK 2 – LED TEMPERATURE MONITORING DEVICE IMPLEMENTATION [25 MARKS]

Main

clear; clc; close all;

% Hardware configuration - Arduino & pin definitions

arduino_port = 'COM3';       % Arduino serial port

arduino_board = 'Uno';       % Arduino board model

pin_green = 'D2';            % Green LED digital pin (18-24°C range)

pin_yellow = 'D3';           % Yellow LED digital pin (<18°C)

pin_red = 'D4';              % Red LED digital pin (>24°C)

sensor_pin = 'A1';           % Temperature sensor analog pin

% MCP9700A sensor calibration parameters

V0C = 0.4;                   % Voltage at 0°C (V)

Tc = 0.01953;                % Temperature coefficient (V/°C)

monitor_duration = 600;      % Monitoring duration (10 mins = 600s)

% Clear existing Arduino connection to avoid conflict

if exist('a', 'var')

clear a;

disp(' The old Arduino connection has been cleared. ');

end

% Establish Arduino communication (critical for hardware control)

try

a = arduino(arduino_port, arduino_board);  % Create Arduino communication object

disp(' The Arduino connection was successful!');

catch ME

error('Arduino connection failed.');

end

disp([' Start the temperature monitoring system... (Duration: ', num2str(monitor_duration/60), 'min)']);

% Execute temperature prediction function (Task 3 core logic)

try

temp_prediction(a, pin_green, pin_yellow, pin_red, sensor_pin, V0C, Tc, monitor_duration);

catch ME

disp([' Monitoring interrupted/error: ', ME.message]);

end

% Cleanup - turn off all LEDs and release Arduino connection

if exist('a', 'var')

writeDigitalPin(a, pin_green, 0);  % Set pin to low (turn off LED)

writeDigitalPin(a, pin_yellow, 0);

writeDigitalPin(a, pin_red, 0);

clear a;

disp(' Resource cleanup completed! ');

else

disp(' There is no valid Arduino connection. No need to clean ');

end

function temp_monitor

function temp_monitor(a, pin_green, pin_yellow, pin_red, sensor_pin, V0C, Tc, duration)

configurePin(a, pin_green, 'digitaloutput');  % Set green LED pin as digital output

configurePin(a, pin_yellow, 'digitaloutput'); % Set yellow LED pin as digital output

configurePin(a, pin_red, 'digitaloutput');    % Set red LED pin as digital output

% Initialize all LEDs to off (low level)

writeDigitalPin(a, pin_green, 0);

writeDigitalPin(a, pin_yellow, 0);

writeDigitalPin(a, pin_red, 0);

% Create real-time plot window

figure('Color','w', 'Position', [100, 100, 800, 500]);

h_plot = plot(0, 0, 'b-o', 'LineWidth', 1.5, 'MarkerSize', 4);

xlabel('Time (s)', 'FontSize', 12);

ylabel('Temperature (°C)', 'FontSize', 12);

title(['Real-time Temperature Monitoring (MCP9700A) - ', num2str(duration/60), 'minute'], 'FontSize', 14);

grid on;

xlim([0, duration]);

ylim([0, 40]);

legend('Temperature', 'Location', 'best');

time_list = [];  % Store time data for plot

temp_list = [];  % Store temperature data for plot

start_time = tic; % Start timing

while true

V_out = readVoltage(a, sensor_pin);  % Read sensor output voltage (A1 pin)

temp = (V_out - V0C) / Tc;          % Convert voltage to temperature (MCP9700A formula)

current_time = toc(start_time);     % Get elapsed time

time_list = [time_list, current_time];

temp_list = [temp_list, temp];

% Update real-time plot data and refresh

set(h_plot, 'XData', time_list, 'YData', temp_list);

xlim([0, max(current_time, duration)]);

ylim([min(temp_list)-2, max(temp_list)+2]);

drawnow;

% LED control logic based on temperature range

if temp >= 18 && temp <= 24

writeDigitalPin(a, pin_green, 1);  % Green LED on (comfort range: 18-24°C)

writeDigitalPin(a, pin_yellow, 0);

writeDigitalPin(a, pin_red, 0);

pause(1);

elseif temp < 18

writeDigitalPin(a, pin_green, 0);

writeDigitalPin(a, pin_red, 0);

writeDigitalPin(a, pin_yellow, 1); % Yellow LED blink (0.5s on/off)

pause(0.5);

writeDigitalPin(a, pin_yellow, 0);

pause(0.5);

else

writeDigitalPin(a, pin_green, 0);

writeDigitalPin(a, pin_yellow, 0);

writeDigitalPin(a, pin_red, 1);    % Red LED blink (0.25s on/off)

pause(0.25);

writeDigitalPin(a, pin_red, 0);

pause(0.25);

end

% Stop monitoring when reaching set duration

if current_time >= duration

disp([' The monitoring duration has reached（', num2str(duration/60), 'minutes）,Stop automatically. ']);

break;

end

fprintf(' %.1fs | temperature: %.2f ℃\n', current_time, temp);

end

% Turn off all LEDs after monitoring ends

writeDigitalPin(a, pin_green, 0);

writeDigitalPin(a, pin_yellow, 0);

writeDigitalPin(a, pin_red, 0);

disp('All leds have been turned off and the monitoring has ended normally!');

end

Touch the sensor

Stop touching

TASK 3 – ALGORITHMS – TEMPERATURE PREDICTION [ 30 MARKS]

Main

clear; clc; close all;

arduino_port = 'COM3';

arduino_board = 'Uno';

pin_green = 'D2';

pin_yellow = 'D3';

pin_red = 'D4';

sensor_pin = 'A1';

V0C = 0.4;

Tc = 0.01953;

monitor_duration = 600

if exist('a', 'var')

clear a;

disp(' The old Arduino connection has been cleared. ');

end

try

a = arduino(arduino_port, arduino_board);

disp(' The Arduino connection was successful!');

catch ME

error('Arduino connection failed.');

end

disp([' Start the temperature monitoring system... (Duration: ', num2str(monitor_duration/60), 'min)']);

try

temp_prediction(a, pin_green, pin_yellow, pin_red, sensor_pin, V0C, Tc, monitor_duration);

catch ME

disp([' Monitoring interrupted/error: ', ME.message]);

end

if exist('a', 'var')

writeDigitalPin(a, pin_green, 0);

writeDigitalPin(a, pin_yellow, 0);

writeDigitalPin(a, pin_red, 0);

clear a;

disp(' Resource cleanup completed! ');

else

disp(' There is no valid Arduino connection. No need to clean ');

end

function temp_ prediction

function [time_data, temp_data, rate_data, predict_data] = temp_prediction(a, pin_green, pin_yellow, pin_red, sensor_pin, V0C, Tc, duration)

configurePin(a, pin_green, 'digitaloutput');  % Set green LED pin as digital output

configurePin(a, pin_yellow, 'digitaloutput'); % Set yellow LED pin as digital output

configurePin(a, pin_red, 'digitaloutput');    % Set red LED pin as digital output

% Initialize all LEDs to off (low level)

writeDigitalPin(a, pin_green, 0);

writeDigitalPin(a, pin_yellow, 0);

writeDigitalPin(a, pin_red, 0);

window_size = 5;  % Sliding window size for noise reduction (5 samples)

temp_history = [];

time_history = [];

start_time = tic; % Start timing

% Initialize output arrays to store monitoring data

time_data = [];

temp_data = [];

rate_data = [];

predict_data = [];

% Create dual-subplot window for temp & change rate

figure('Color','w', 'Position', [100, 100, 800, 600]);

subplot(2,1,1);

h_temp = plot(0, 0, 'b-o', 'LineWidth', 1.5, 'MarkerSize', 4);

xlabel('Time (s)', 'FontSize', 12);

ylabel('Temperature (°C)', 'FontSize', 12);

title('Real-time Temperature & Change Rate (MCP9700A)', 'FontSize', 14);

grid on;

xlim([0, duration]);

ylim([0, 40]);

legend('Temperature', 'Location', 'best');

subplot(2,1,2);

h_rate = plot(0, 0, 'r-*', 'LineWidth', 1.5, 'MarkerSize', 4);

xlabel('Time (s)', 'FontSize', 12);

ylabel('Change Rate (°C/min)', 'FontSize', 12);

grid on;

xlim([0, duration]);

ylim([-10, 10]);

legend('Change Rate', 'Location', 'best');

while true

V_out = readVoltage(a, sensor_pin);  % Read sensor voltage (A1 pin)

current_temp = (V_out - V0C) / Tc;  % Convert voltage to temperature (MCP9700A formula)

current_time = toc(start_time);     % Get elapsed time

% Update sliding window (keep latest N samples for noise reduction)

temp_history = [temp_history, current_temp];

time_history = [time_history, current_time];

if length(temp_history) > window_size

temp_history(1) = [];

time_history(1) = [];

end

% Calculate temperature change rate (°C/s → °C/min)

if length(temp_history) >= 2

delta_T = temp_history(end) - temp_history(1);

delta_t = time_history(end) - time_history(1);

rate_Cs = delta_T / delta_t;          % Change rate (°C/s)

rate_Cmin = rate_Cs * 60;            % Convert to °C/min

else

rate_Cs = 0;

rate_Cmin = 0;

end

predict_temp = current_temp + rate_Cs * 300;  % Predict temp in 5 mins (300s)

% Update data arrays

time_data = [time_data, current_time];

temp_data = [temp_data, current_temp];

rate_data = [rate_data, rate_Cmin];

predict_data = [predict_data, predict_temp];

% LED alert logic based on change rate (threshold: ±4°C/min)

if rate_Cmin > 4

writeDigitalPin(a, pin_green, 0);

writeDigitalPin(a, pin_yellow, 0);

writeDigitalPin(a, pin_red, 1);    % Red LED on (temp rising too fast)

elseif rate_Cmin < -4

writeDigitalPin(a, pin_green, 0);

writeDigitalPin(a, pin_yellow, 1); % Yellow LED on (temp dropping too fast)

writeDigitalPin(a, pin_red, 0);

else

writeDigitalPin(a, pin_green, 1);  % Green LED on (stable temp)

writeDigitalPin(a, pin_yellow, 0);

writeDigitalPin(a, pin_red, 0);

end

% Update real-time plots

set(h_temp, 'XData', time_data, 'YData', temp_data);

xlim(h_temp.Parent, [0, max(current_time, duration)]);

ylim(h_temp.Parent, [min(temp_data)-2, max(temp_data)+2]);

set(h_rate, 'XData', time_data, 'YData', rate_data);

xlim(h_rate.Parent, [0, max(current_time, duration)]);

drawnow;

% Print real-time data to console

fprintf(' %.1fs | Now：%.2f ℃ | rate of change：%.2f ℃/min | 5min Forecast：%.2f ℃\n', ...

current_time, current_temp, rate_Cmin, predict_temp);

% Stop loop when reaching set duration

if current_time >= duration

disp([' The monitoring duration has reached(', num2str(duration/60), 'min), automatic stop']);

break;

end

pause(1);  % 1s sampling interval

end

% Turn off all LEDs after monitoring

writeDigitalPin(a, pin_green, 0);

writeDigitalPin(a, pin_yellow, 0);

writeDigitalPin(a, pin_red, 0);

disp(' All leds have been turned off and the monitoring has ended normally！');

disp(' Real-time data has been retained and can be viewed through the output parameters');

disp('   - time_data：Sampling time (seconds)');

disp('   - temp_data：Real-time temperature (℃)');

disp('   - rate_data：Real-time rate of change (℃/min)');

disp('   - predict_data：5-minute predicted temperature (℃)');

end

Answer (1 minute):

0.2s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

1.4s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

2.5s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

3.7s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

4.8s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

6.0s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

7.1s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

8.3s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

9.5s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

10.6s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

11.8s | Now：17.81 ℃ | rate of change：3.24 ℃/min | 5min Forecast：34.00 ℃

12.9s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

14.1s | Now：17.81 ℃ | rate of change：3.26 ℃/min | 5min Forecast：34.08 ℃

15.2s | Now：17.81 ℃ | rate of change：3.26 ℃/min | 5min Forecast：34.12 ℃

16.4s | Now：17.81 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.81 ℃

17.5s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

18.7s | Now：17.56 ℃ | rate of change：-3.27 ℃/min | 5min Forecast：1.22 ℃

19.8s | Now：17.56 ℃ | rate of change：-3.27 ℃/min | 5min Forecast：1.23 ℃

21.0s | Now：17.81 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.81 ℃

22.1s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

23.3s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

24.4s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

25.6s | Now：17.56 ℃ | rate of change：-3.29 ℃/min | 5min Forecast：1.12 ℃

26.7s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

27.9s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

29.0s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

30.2s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

31.3s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

32.5s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

33.6s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

34.8s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

35.9s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

37.1s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

38.2s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

39.4s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

40.5s | Now：17.81 ℃ | rate of change：3.26 ℃/min | 5min Forecast：34.09 ℃

41.7s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

42.8s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

44.0s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

45.1s | Now：17.56 ℃ | rate of change：-3.28 ℃/min | 5min Forecast：1.18 ℃

46.3s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

47.4s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

48.6s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

49.7s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

50.9s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

52.0s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

53.2s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

54.3s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

55.5s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

56.6s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

57.8s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

58.9s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃

60.1s | Now：17.56 ℃ | rate of change：0.00 ℃/min | 5min Forecast：17.56 ℃